RapidSmith 2

A Library for Low-level Manipulation 
of Vivado Designs at the Cell/BEL Level

Technical Report and Documentation

<Authors>

NSF Center for High Performance Reconfigurable Computing (CHREC)
Department of Electrical and Computer Engineering
Brigham Young University
Provo, UT 84602

Revised: 12 July 2016

This work was supported in part by the I/UCRC program of the National Science Foundation, grant number 1265957.

# Introduction

## What is RapidSmith 2?

The original BYU RapidSmith project began in 2010 with the goal to develop a set of tools and APIs written in Java which would provide academics with an easy-to-use platform to try out experimental CAD ideas and algorithms on modern Xilinx FPGAs.  RapidSmith 2 (abbreviated RS2 hereafter) represents a major addition to RapidSmith.  Using RS2 you can write custom CAD tools which will:

- export designs from Vivado

- perform analyses on those designs

- make modifications to those designs

- import those designs back into Vivado for further processing or bitstream generation

In addition, you need not start with a Vivado design – you can create a new design from scratch in RS2 and then import it into Vivado.

The major new capability which RS2 adds over RapidSmith is that it changes RapidSmith’s design representation from the Instances and Sites of ISE’s XDL language to the Cells and BELs of Vivado.

This opens up a world of new CAD research opportunities which were difficult to perform using Rapidsmith.

## Who Should Use RS2?

RS2 is aimed at anyone desiring to do FPGA CAD research on real Xilinx devices.  It is written in Java. It also depends on some understanding of Xilinx FPGAs, Vivado, and TCL.  However, the goal is that this documentation provides sufficient background and detail to help bring developers up to speed on the needed topics.

RS2 by no means is a Xilinx Vivado replacement and cannot be used without a valid and current license to a Xilinx tools installation.

## Why RS2?

The Xilinx-provided TCL interface into Vivado, in theory, provides all that is needed to create any kind of CAD tool desired to augment the capabilities provided by Vivado.  In practice there are a number of problems with that. First, TCL is slow, far too slow to execute a router for example.  Also the Xilinx TCL interface does not manage memory well.  In our experience, long running scripts eventually cause the system to run out of memory (this has been unofficially confirmed by reading between the lines of the responses we have received to bug reports we have filed).  Brad White’s MS work also determined that not 100% of the device information required to do arbitrary CAD manipulations is available through TCL.  As a result, additional tools (and some small amount of manual work) are required to provide the user (and CAD tools they might like to write) with all the physical details on Xilinx parts (simply put, some information is not available through the TCL interface).  Finally, the ability to export and import designs to/from Vivado and operate on them outside Vivado using a modern high-level language such as Java is a hugely useful capability.

RS2 (in conjunction with Tincr which is described in a later section of this document) takes care of all of the generation of the FPGA part information that is required by CAD tools. It also takes care of exporting/importing designs from and to Vivado along with a myriad number of fairly arcane details associated with that process.  In addition, RS2 creates special device files from the XDLRC files produced by Tincr and provides a nice API into those physical device details.  All of this enables researchers to have more time to focus on what matters most: their research of new ideas and algorithms.

## Which Xilinx Parts does RS2 Support?

As of the writing of this document, Virtex 7 has been tested the most and are currently supported in all forms and applications.  In addition, an Ultrascale device file was created and demonstrated as a part of Brad White’s MS work and, at some point, Ultrascale should be fully supported.

As will be seen later, to generate additional device files for additional parts within a supported family is relativel straightforward and can be done by any user.  As will also be seen later, new families can also be supported but this requires more work.

## How is RS2 Different than VPR and VTR?

VPR (Versatile Place and Route) has been an FPGA research tool for several years and has led to hundreds of publications on new FPGA CAD research.  It has been a significant contribution to the FPGA research community and has grown to be a complete FPGA CAD flow for research-based FPGAs.

The main difference between RapidSmith/RS2 and VPR is that the RapidSmith tools aim to provide the ability to target commercial Xilinx FPGAs, providing the ability to exit and re-enter the standard Xilinx flow at any point. All features of commercial FPGAs which are accessible via XDL and Vivado’s TCL are available in RapidSmith and RS2.  Our understanding is that VPR currently is limited to FPGA features which can be described using VPR's architectural description facilities.

## Why Java?

We have found Java to be a rapid prototyping platform for FPGA CAD tools.  The Java libraries are rich with data structures useful for such applications and Java eliminates the need to clean up objects in memory.  This eliminates the time needed to debug such things in other development platforms, leaving more time for the researcher to focus on the real research at hand.  Our experience over the past decade is that for student research projects, the lack of memory management and its associated errors has greatly improved our student productivity and led to far more stable CAD tools.

# Vivado, RS2, and Tincr

## RapidSmith vs. RS2

### What Was The Original RapidSmith?

The original RapidSmith was written by Christopher Lavin as a part of his PhD work at BYU.  It was based on the Xilinx Design Language (XDL) which provides a human-readable file format equivalent to the Xilinx proprietary Netlist Circuit Description (NCD) of ISE.  With RapidSmith, researchers were able to import XDL/NCD, manipulate, place, route and export designs among a variety of design transformations.  The RapidSmith project made an excellent test bed to try out new ideas and algorithms for FPGA CAD research as code could quickly be written to take advantage of the APIs available.

RapidSmith also contained packages which could parse/export bitstreams (at the packet level) and represent the frames and configuration blocks in the provided data structures.  In this regard, RapidSmith did not include any proprietary information about Xilinx FPGAs that is not publicly available.

RapidSmith continues to be functional and is still available at the SourceForge.net website.  There, you will find documentation, installation instructions, the RapidSmith code base, and a collection of demo programs based on it.

### What is RS2?

With the announced end of ISE (with the Virtex7 family of parts being the last family to be supported by ISE), there was no path forward to newer parts using RapidSmith.  This is because XDL is not available with Vivado. However, with Vivado Xilinx has provided an extensive TCL scripting capability which it initially looked as if it could provide a similar capability to that provided by XDL in terms of accessing both Vivado’s design and device data and in terms of creating and modifying Vivado designs.  The development of RS2 consisted of two parts.

#### Tincr: Integrating Custom CAD Tool Frameworks with the Xilinx Vivado Design Suite

In the first part, the Vivado TCL capability was investigated to ensure that, indeed, it did provide the needed ability to access design and device data and export that to external tools such as RapidSmith.  This resulted in the Tincr project, led by Brad White as a part of his MS work at BYU with Thomas Townsend making additions as a part of his research.

Tincr is a TCL-based library of routines which (a) provide a variety of functions to simply make working with Vivado via TCL simpler, (b) provide a way to export all the data associated with a Vivado design into what is called a Tincr Checkpoint (TCP), (c) provide a way to reimport Tincr Checkpoints back into Vivado, and (d) access device data from Vivado and output that data in the form of XDLRC files (these are the files which XDL used to describe devices and are necessary for RapidSmith to understand the structure of and the resources  available for use in a given Xilinx part.  Tincr is available at Github.com as the project byuccl/Tincr.  Tincr is described in two publications:

B. White and B. Nelson, "Tincr — A custom CAD tool framework for Vivado," 2014 International Conference on ReConFigurable Computing and FPGAs (ReConFig14), Cancun, 2014, pp. 1-6, DOI: 10.1109/ReConFig.2014.7032560

White, Brad S., "Tincr: Integrating Custom CAD Tool Frameworks with the Xilinx Vivado Design Suite" (2014), BYU Scholars Archive, Paper 4338.  URL: http://scholarsarchive.byu.edu/etd/4338

#### RS2: A Framework for BEL-Level CAD Exploration on Xilinx FPGAs

The second part of the development of RS2 was to add a new layer of design representation to RapidSmith which more closely matches that of Vivado.  This was done as a part of his PhD work by Travis Haroldsen at BYU.  As of this writing, an initial paper on RS2 has appeared:

Travis Haroldsen, Brent Nelson, and Brad Hutchings, “RapidSmith 2: A Framework for BEL-Level CAD Exploration on Xilinx FPGAs”, Proceedings of the 2015 ACM/SIGDA International Symposium on Field-Programmable Gate Arrays, February 2015, Monterey CA, pp. 66-69, DOI: 10.1145/2684746.2689085.

### What is All This About XDL and How Does RS2 Fit Into That?

The Xilinx ISE tools had the capability to export XDL and XDLRC files which RapidSmith used:

- An XDLRC file was a complete description of a given Xilinx FPGA, describing every tile, every switchbox, every wire segment, and every PIP in the part.  Rapidsmith was able to process this information and use it to support the creation of CAD tools such as placers and routers.

- An XDL file was a textual representation of an NCD file (a user design).  It described the user design as a collection of Instances and Nets. Instances correspond to things like SLICEs, BRAMs, DSP48s, and IOBs.  Instances could be placed onto primitive sites. Additionally, Nets in XDL consisted of a list of pins (their logical connections) and an optional list of PIPs (their physical routing connections).

In Vivado, however, designs are described as a collection of Cells where a cell corresponds to things like LUTs, flip flops, etc.  Cells are placed onto BEL objects such as an ALUT or a BFF.  RS2 contains a new layer of hierarchy in its design and device descriptions where Cells and BELs are first-class objects and design manipulation is all done at the Cell/BEL level.

Also, Vivado Nets are described using directed routing strings rather than lists of PIPs.  RS2 also contains a set of new classes to enable the representation and manipulation of Nets using these routing strings.

Thus, using RS2, design manipulation is now done at the level of Cells and BELs and importing/exporting designs to/from Vivado is now fully supported.

## RS2 Usage Model and Structure

The usage model for RS2 is shown in Figure 1.  As can be seen, a design can be exported from Vivado at multiple different points in the Vivado design flow.  In each case, Tincr is used to export a Tincr Checkpoint which can then be imported into RS2.  At those same points in the design flow, RS2 can export a Tincr Checkpoint which can then be imported back into Vivado.  Thus, a complete solution involves Vivado, Tincr, and RS2.

Figure 1 – Vivado and RS2

# Getting Started

## Installation

### Getting RS2

RS2 is available on Github at:

https://github.com/byuccl/RapidSmith2

The repository contains all the files you need (including supporting JAR files).  While project files are not included in the Github repo, we highly recommend you use something like Eclipse as your IDE.

### Requirements for Installation

- Windows, Linux or Mac OS X all will work (see notes below for Mac OS X)

- Vivado

- JDK 1.8 or later NOTE: If you plan on using the Qt Jambi framework in a Windows environment, you will need the 32-bit JRE (Qt Jambi 4.6.3 had yet to be compiled in 64-bit Windows as of the writing of this document).

- Supporting JARs

  - INCLUDED: Caucho Hessian Implementation JAR v.4.0.6 (Used for compressing database device files)

  - INCLUDED: Qt Jambi (Qt for Java) for the Part Tile Browser example.  Just adding the jars to the CLASSPATH variable is adequate.

  - INCLUDED: JOpt Simple for use by some examples in the bitstream tools packages.

  - OPTIONAL: JavaCC if the user wants to change the XDL design parser. There is also a good plugin for Eclipse for JavaCC which makes it easier to modify and compile .jj files.

### Steps for Installation For Command Line Usage

- Clone the Github byuccl/RapidSmith2 project to a local place.  This can be done by executing the  following command from the command line of a machine with git installed: 
               git clone https://github.com/xrtc/RapidSmith2
This will create a clone of the Github repository for RapidSmith2 in the current directory (it will create a RapidSmith2 directory and put it into that).

- Add all the jar files in the jars folder (hessian-4.0.6.jar, qtjambi-4.6.3.jar, qtjambi-<your_platform>-4.6.3.jar, jopt-simple-3.2.jar) to your CLASSPATH environment variable.

- Add the location of your Java project to your CLASSPATH environment variable.

- Create an environment variable called RAPIDSMITH_PATH and set its value to the path where you have the Java project located on your computer.

- Compile all of the Java classes.

- Test your installation by running any of the programs such as:

java edu.byu.ece.rapidSmith.device.browser.DeviceBrowser

If all goes well you should see a graphical representation showing the details of a physical FPGA device as shown in Figure 2.

Figure 2 – Device Browser Sample Display

### Steps for Installation For Eclipse Usage

For Eclipse usage, you follow essentially the same steps above.  The major difference is that you perform each task via Eclipse commands:

- Using FileImport from Eclipse you can clone the Github repo at byuccl/RapidSmith2 locally and then using the New Project Wizard create an Eclipse project for the cloned copy of RapidSmith2.  IMPORTANT: do not specify a location inside your Eclipse workspace as the destination for the Import – Eclipse does not like that and will give vague errors if you do so.  Instead, import into a location like /Users/smith/Documents/git/RapidSmith2.   As above, the location to clone from is https://github.com/xrtc/RapidSmith2.

- Add all the jar files in the jars folder (hessian-4.0.6.jar, qtjambi-4.6.3.jar, qtjambi-<your_platform>-4.6.3.jar, jopt-simple-3.2.jar) to your project in Eclipse.

- Define an environment variable called RAPIDSMITH_PATH and set its value to the path where you have the Java project located on your computer.  This can be done globally on your computer or it can be defined in each Run Configuration you create.

- Test your installation by creating a Run Configuration to execute the program:

	edu.byu.ece.rapidSmith.device.browser.DeviceBrowser 

and it should bring up the GUI above in Figure 2

### Device Files For Use With RS2

Device files for one part (the xc7a100tcsg324) are included in the repo so you can immediately start working with RS2 using this part (initially, it will be the only device available when you run the DeviceBrowser above).  If you desire to work with additional parts, follow the instructions found in this documentation in Section 7 Generating Device Files in RS2 on page 20.

### Additional Notes for Mac OS X Installation

- The instructions above require you to set environment variables.  If using an IDE such as Eclipse, this can be accomplished inside Eclipse (see Run configurations, Environment tab).   Adding environment variables can also be done directly in Mac OS X.  To add/modify global environment variables in Mac OS X, one preferred way would be to edit the environment.plist file in the ~/.MacOSX directory.  Here is an example of a proper setup for RapidSmith (assuming the location where the RS2 repo was cloned to):

<?xml version="1.0" encoding="UTF-8"?> <!DOCTYPE plist PUBLIC "-//Apple//DTD PLIST 1.0//EN" "http://www.apple.com/DTDs/PropertyList-1.0.dtd"> <plist version="1.0"> <dict> <key>RAPIDSMITH_PATH</key> <string>/Users/[user name]/Documents/git/RapidSmith2</string> <key>CLASSPATH</key> <string>$CLASSPATH:/Users/[user name]/Documents/git/RapidSmith2:/Users/[user name]/Documents/git/RapidSmith2:/Users/[user name]/Documents/git/RapidSmith2/jars/hessian-4.0.6.jar:/Users/[user name]/Documents/git/RapidSmith2/jars/qtjambi-4.6.3.jar:/Users/[user name]/Documents/git/RapidSmith2/jars/qtjambi-macosx-gcc-4.6.3.jar</string> </dict> </plist>

- Another difference is that when running programs that use Qt in Java under Mac OS X, the user will need to supply an extra JVM switch, “-XstartOnFirstThread”

# Overview of RS2 And Its Packages

RS2 is a superset of RapidSmith.  In the future some features of RapidSmith which have been super ceded by RS2 may be removed. Initially, however, the RS2 code base is a proper superset of RapidSmith – all of RapidSmith’s code is included in the RS2 distribution. As a result, included in the Github release is code that is either (a) super ceded by RS2 functionality or (b) no longer functions or is needed but has not been removed yet.

The subsite package is an example of (a) type code – it is intended that designs now be created and modified using that package’s functionality rather than that found in the original RapidSmith design package.  An example of (b) type code are some of the included example applications – many of them work in RS2, some may not.  However, none will use new RS2 features.   All of this will likely change as RapidSmith evolves – the discussion above describes RS2 at the time of release on Github.

RapidSmith is organized into several packages (all packages are prefixed with “edu.byu.ece.rapidSmith”).  Some packages are labelled below with RapidSmith or RS2 to indicate if they are specific to one or the other.

## The bitstreamTools.* Package
(RapidSmith, deprecated)

Represents the packet view of a Xilinx bitstream.  It contains classes to represent the header, packets, types, configuration registers and bitstream parsing and export facilities.  Contains a number of sub-packages such as (bitstreamTools.configuration).

NOTE: this does not contain details on what the individual bitstream bits mean, rather it is a set of utilities which help at the bitstream packet level.  Has not be tested beyond Virtex6 – unclear how it might or might not work with newer parts.

Please see the chapter on Bitstreams in RapidSmith in the original RapidSmith documentation for details on the bitstream functionality in RapidSmith.  It has not been changed for RS2 and may or may not work in RS2.

## The design.* Package
(RapidSmith, deprecated)

Deprecated.  Replaced by the subsite package.  Please see the original RapidSmith documentation for details.

## The subsite Package
(RS2)

Represents all of the constructs in Vivado designs (Cells, CellDesigns,  CellNets, LibraryCells, and RouteTrees).  This is what makes the RS2 data representation compatible with Vivado’s data representation.  Replaces RapidSmith’s design package.

## The device Package
(RS2)

Updated for use in RS2.

This package works closely with the design package in that the specific Device class is loaded when a design is loaded.  The part descriptions partition the FPGA into a 2D grid of tiles.  Each tile contains some mixture of primitive sites, wires and PIPs (Programmable Interconnect Points).  Primitive sites are resource locations which contain BELs.  Wires and PIPs provide wiring and routing resources information to connect the primitive instances together to form a complete design.  With this information provided by Xilinx and leveraged by RS2 a number of different placement and routing algorithms can be constructed by leveraging the APIs in this package.

## The device.browser Package
(RS2)

Updated for use in RS2.

The device browser application allows the user to see a color coded tile array that allows them to browse any installed device.  The primitive and wire lists are populated by double clicking a tile.  The user can zoom in and out using the mouse wheel and can also pan by holding down the right mouse button while moving the mouse.

All that is required for this to operate is a valid device file (no design required).  See below for a screenshot of the application.

Figure 3 - Device Browser screenshot

The device browser also allows the user to follow the various connections found in the FPGA.  By double clicking a wire in the wire list, the application will draw the connection on the tile array (as shown in the screenshot below).  By hovering the mouse pointer over the connection, the wire becomes red and a tooltip will appear describing the connection made by declaring the source tile and wire followed by an arrow (->) and the destination tile and wire.  By clicking on the wire, the application will redraw all the connections that can be made from the currently selected wire.  By repeating this action, the user can follow connections and discover how the FPGA interconnect is laid out.

Figure 4 - Device browser screenshot showing wire connections

### The device.helper Package
(???)

Note clear if this works or is needed in RS2.

This package contains special classes to help pack the device files smaller.  They are generally used only during installation.

## The examples Package
(RapidSmith and RS2)

This package contains some examples of how to get started with RapidSmith and some different ways of using the various APIs available. Some work with RS2 but none updated specifically for it.  Those that work will not make use of any of the new RS2 capabilities.  Currently unclear which ones will or will not work with RS2

## The examples2 Package
(RS2)

This package contains some RS2-specific examples.  As programs from the examples package are determined to fully work with RS2 or are ported to use RS2 features, our hope is that they will be migrated to this package.

## The placer Package
(RapidSmith, deprecated)

Deprecated. Please see the original RapidSmith documentation for details (never was fully implemented in RapidSmith).

## The primitiveDefs Package
(RapidSmith, deprecated)

In the XDLRC descriptions produced by the Xilinx ‘xdl’ executable, each copy has a section at the end called primitive_defs which has a list of primitive definitions for all types of primitives found in the part. The primitiveDefs packages makes that information available in a convenient data structure to access the attributes and various parameters the primitives can be configured with.  However, RapidSmith never used that information.

RS2 is all about this information as it is what allows RS2 to work at the Cell and BEL level rather than just at the Instance and Slice level.  Thus, this package has been replaced by RS2’s subsite package.

## The router Package
(RapidSmith, deprecated)

Deprecated. Please see the original RapidSmith documentation for details.

## The tests Package
(???)

What is the status of this?

In order to help ensure correct functionality in RapidSmith as it grows, a tests package has been added to hold all of the different tests that can be performed to check for correct functionality with each new update.  Currently, this package contains a class for testing the device, primitive defs and wire enumerator files (it is the class used to create the statistical information on RapidSmith files found in the Appendix).

## The timing Package
(RapidSmith, deprecated)

Deprecated. Please see the original RapidSmith documentation for details.

## The util Package
(RapidSmith and RS2)

This has miscellaneous classes used for support of all other packages.  It is suggested to have the user browse the JavaDoc API descriptions to get a better feel for what is contained in the util package.  Some parts may work with RS2 and some may not.

# Examples

## Hello World

To get started programming with RapidSmith, here is an example of a very simple program.

Create equivalent in RS2

/* * Copyright (c) 2010 Brigham Young University * * This file is part of the BYU RapidSmith Tools. * * BYU RapidSmith Tools is free software: you may redistribute it * and/or modify it under the terms of the GNU General Public License * as published by the Free Software Foundation, either version 2 of * the License, or (at your option) any later version. * * BYU RapidSmith Tools is distributed in the hope that it will be * useful, but WITHOUT ANY WARRANTY; without even the implied warranty * of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU * General Public License for more details. * * A copy of the GNU General Public License is included with the BYU * RapidSmith Tools. It can be found at doc/gpl2.txt. You may also * get a copy of the license at <http://www.gnu.org/licenses/>. * */ package edu.byu.ece.rapidSmith.examples; import java.util.HashMap; import edu.byu.ece.rapidSmith.design.*; import edu.byu.ece.rapidSmith.device.*; /** * A simple class to illustrate how to use some of the basic methods in RapidSmith. * @author Chris Lavin */ public class HelloWorld{ public static void main(String[] args){ // Create a new Design from scratch rather than load an existing design Design design = new Design(); // Set its name design.setName("helloWorld"); // When we set the part name, it loads the corresponding Device and // WireEnumerator. Always include package and speed grade with the part name. design.setPartName("xc4vfx12ff668-10"); // Create a new instance Instance myInstance = new Instance(); myInstance.setName("Bob"); myInstance.setType(PrimitiveType.SLICEL); // We need to add the instance to the design so it knows about it design.addInstance(myInstance); // Make the F LUT an Inverter Gate myInstance.addAttribute(new Attribute("F","LUT_of_Bob","#LUT:D=~A1")); // Add the instance to the design design.addInstance(myInstance); // This is how we can get the reference to the instance from the design, // by name Instance bob = design.getInstance("Bob"); // Let's find a primitive site for our instance Bob HashMap<String, PrimitiveSite> primitiveSites = design.getDevice().getPrimitiveSites(); for(PrimitiveSite site : primitiveSites.values()){ // Some primitive sites can have more than one type reside at the site, such as // SLICEM sites which can also have SLICELs placed there. Checking if the site // is compatible makes sure you get the best possible chance of finding a place // for bob to live. if(site.isCompatiblePrimitiveType(bob.getType())){ // Let's also make sure we don't place bob on a site that is already used if(!design.isPrimitiveSiteUsed(site)){ bob.place(site); System.out.println("We placed bob on tile: " + bob.getTile() + " and site: " + bob.getPrimitiveSiteName()); break; } } } // Another way to find valid primitive sites if we want to use an exclusive site type PrimitiveSite[] allSitesOfTypeSLICEL = design.getDevice().getAllSitesOfType(bob.getType()); for(PrimitiveSite site : allSitesOfTypeSLICEL){ // Let's also make sure we don't place bob on a site that is already used if(!design.isPrimitiveSiteUsed(site)){ bob.place(site); System.out.println("We placed bob on tile: " + bob.getTile() + " and site: " + bob.getPrimitiveSiteName()); break; } } // Let's create an IOB to drive our Inverter gate in Bob's LUT Instance myIOB = new Instance(); myIOB.setName("input"); myIOB.setType(PrimitiveType.IOB); design.addInstance(myIOB); // These are typical attributes that need to be set to configure the IOB // the way you like it myIOB.addAttribute(new Attribute("INBUFUSED","","0")); myIOB.addAttribute(new Attribute("IOATTRBOX","","LVCMOS25")); // Another way to find a primitive site is by name, this is the pin name // that you might find in a UCF file myIOB.place(design.getDevice().getPrimitiveSite("C17")); // Let's also create a new net to connect the two pins Net fred = new Net(); // Be sure to add fred to the design design.addNet(fred); fred.setName("fred"); // All nets are normally of type WIRE, however, some are also GND and VCC fred.setType(NetType.WIRE); // Add the IOB pin as an output pin or the source of the net fred.addPin(new Pin(true,"I",myIOB)); // Add Bob as the input pin or sink, which is the input to the inverter fred.addPin(new Pin(false, "F1", bob)); // Now let's write out our new design // We'll print the standard XDL comments out String fileName = design.getName() +".xdl"; design.saveXDLFile(fileName, true); // We can load XDL files the same way. Design inputFromFile = new Design(); inputFromFile.loadXDLFile(fileName); // Hello World System.out.println(inputFromFile.getName()); } }

## Hand Router

What is this subsection talking about?

This is a command-line-based router than allows a user to route one net at a time and write out the design changes afterwards.  Although not particularly useful as a router, it illustrates how RapidSmith could be used to build a router.

## Part Tile Browser

I think this is talking about the Device Browser?  There ought to at least be instructions on how to run it.  There are none.

This GUI will let you browse Virtex 4 and 5 parts at the tile level.  On the left, the user may choose the desired part by navigating the tree menu and double-clicking on the desired part name.  This will load the part in the viewer pane on the right (the first available part is loaded at startup).  The status bar in the bottom left displays which part is currently loaded.  Also displayed is the name of the current tile which the mouse is over, highlighted by a yellow outline in the viewer pane. The user may navigate inside the viewer pane by using the mouse.  By right-clicking and dragging the cursor, the user may pan.  By using the scroll-wheel on the mouse, the user may zoom.  If a scroll-wheel is unavailable, the user may zoom by clicking inside the viewer pane and pressing the minus(-) key to zoom out or the equals(=) key to zoom in. See below for a screenshot.

##### Figure 5: Screenshot of Part Tile Browser

# RS2 Structure

Keep this since it can serve as a map to what should be used in RS2.

This section details much of the complexity and theory behind the structure of RS2.  There are two main abstractions that developers need to be aware of; that of the device and that of the design.  A hierarchy of classes within RapidSmith can be seen in Figure 6 below.

##### Figure 6: (a) The classes involved in defining a design in RS2, (b) The major classes involved representing a device.

## Designs in RS2

Designs in RS2 are represented and stored in the data structures found in the subsite package.

Redo.  Ought to contain a nice overview of the subsite package’s support for designs with some examples of use

## Devices in RS2

A device is defined in RS2 as a unique Xilinx FPGA part that includes package information but not speed grade (such as the xc7a100tcsg324 device included in the RS2 distribution).  Each device contains specific information concerning its primitive sites, tiles, wires, IOBs, and PIPs that are available to realize designs.  The device information is represented in RS2 in the device package.  RS2 has significantly extended the original RapidSmith device class for its use as well as how device files are generated.

Device information comes from XDLRC files.  In RapidSmith these were generated by the Xilinx executable xdl in -report mode. In RS2 they are generated by the Tincr package described earlier in the report.  See a later section of this document for information creating device files for more details on creating devices.

Everything below here needs to be rewritten

### Device

During the initial setup of RapidSmith, the Installer creates fully verbose XDLRC files ($xdl –report –pips –all_conns <partName>) for each device specified as command line parameters.  After the creation of each XDLRC file, they are parsed, compacted by the Installer, and a device file is generated for later use.  These device files are placed in $(RAPIDSMITH_PATH)/devices/familyName/partName_db.dat and then the corresponding XDLRC file is deleted as they can be several gigabytes in size.  These device files make accessing device information about a specific FPGA part much more convenient than a gigantic text file.  Most of the device files are just a few megabytes or less and can be loaded in a few seconds or less.  RapidSmith uses a custom form of serialization as well as a compression library to make sure the device files are small and load quickly.

### Wire Enumerator

In order to make the device files small, each uniquely named wire is assigned to an integer as enumeration.  This avoids moving strings around in memory which would be costly in terms of space and comparison times.  RapidSmith has a class called WireEnumerator which enumerates all uniquely named wires in an FPGA family and has methods to convert to and from the wire name and enumeration or enum for short.  It also stores information about each wire such as a direction or type which can be useful in building a router.  Note that wires with the same name can occur several times within a device and they are uniquely identified not only by their name, but also by the tile in which they are present.

In order to create the wire enumeration files, a subset of XDLRC files must be parsed so that a complete set of wires can be enumerated.  This is automatically done by the installer and the files are placed in $(RAPIDSMITH_PATH)/devices/familyName/wireEnumerator.dat.  Only one wire enumerator is needed per FPGA family.

## Memory and Performance in RS2

XDLRC files are huge files (many 10’s of GB in size) and much effort was made in the original RapidSmith to reduce the resulting design representation to manageable sizes.  RS2 benefits from all that work.  See the original RapidSmith document for details on this.

# Generating Device Files in RS2

Needs writing.  Should cover the simpler case of generating a device file for a supported family.  Should also cover the more complex case of creating support for a new family, using V7 or Ultrascale as an example.

# Placement in RS2

This chapter is intended to help users of RS2 understand how placement works in RS2 and in Vivado.

Needs a total rewrite

# Routing in RS2

This chapter is intended to help users and developers in understanding how routing resources are handled in RS2.  It also illustrates how to build on the existing classes to create custom routers.  RS2’s routing handling is significantly different from that found in RapidSmith.

Needs a total rewrite, some can be lifted from the original RapidSmith documentation.

# Bitstreams in RS2

In the original RapidSmith, bitstreams can be parsed, manipulated, and exported for Virtex 4, Virtex 5 and Virtex 6 Xilinx FPGA families.  Because of the proprietary nature of Xilinx bitstreams, RapidSmith provided only documented functionality when working with bitstreams.  This functionality came mainly from the documents distributed by Xilinx in the form of user guides, whitepapers and application notes.  In other words, no proprietary information was used in the creation of the RapidSmith bitstream functionality.

The bitstream functionality from RapidSmith has been left intact in RS2 if users choose to use it with two caveats:

- It has not been tested beyond Virtex 6.

- It is not documented here – for documentation see the original RapidSmith User Guide distributed as a part of the RS2 distribution (in the doc directory).

# Legal and Dependencies

RS2 is a derivative of RapidSmith and is thus covered by the same license:

## RapidSmith Legal Text

BYU RapidSmith Tools

Copyright (c) 2010-2016 Brigham Young University

BYU RapidSmith Tools is free software: you may redistribute it

and/or modify it under the terms of the GNU General Public License

as published by the Free Software Foundation, either version 2 of

the License, or (at your option) any later version.

BYU RapidSmith Tools is distributed in the hope that it will be

useful, but WITHOUT ANY WARRANTY; without even the implied warranty

of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU

General Public License for more details.

A copy of the GNU General Public License is included with the BYU

RapidSmith Tools. It can be found at doc/gpl2.txt. You may also get

a copy of the license at <http://www.gnu.org/licenses/>.

## Included Dependency Projects

RS2 includes the Caucho Technology Hessian implementation which is distributed under the Apache License. A copy of this license is included in the doc directory in the file APACHE2-LICENSE.txt. This license is also available for download at:

http://www.apache.org/licenses/LICENSE-2.0

The source for the Caucho Technology Hessian implementation is available at:

http://hessian.caucho.com

RS2 also includes the Qt Jambi project jars for Windows, Linux and Mac OS X.  Qt Jambi is distributed under the LGPL GPL3 license and copies of this license and exception are also available in the /doc directory in files LICENSE.GPL3.TXT and LICENSE.LGPL.TXT respectively. These licenses can also be downloaded at:

http://www.gnu.org/licenses/licenses.html

Source for the Qt Jambi project is available at:

http://qt.nokia.com/downloads

and more recent versions are available at:

http:/qt.gitorious.org/qt-jambi

RS2 also includes the JOpt Simple option parser which is released under

the open source MIT License which can be found in this directory in the file

MIT_LICENSE.TXT.  A copy of this license can also be found at:

http://www.opensource.org/licenses/mit-license.php

A copy of the source for JOpt Simple can also be downloaded at:

http://jopt-simple.sourceforge.net/download.html

The user is responsible for providing copies of these licenses and making available the source code of these projects when redistributing these jars.

What other dependencies are there now?

# Appendix

Here is just a grouping of useful topics that may not fit in the rest of this document.

## Appendix A: Modifying LUT Content (deprecated)

LUTs (look up tables) found in Xilinx slices can be easily modified using RapidSmith.  LUT content is stored in an attribute in an instance of a SLICEL or SLICEM or whatever type of SLICE the device has.  Often the name of the LUT is a single letter such as F or G as in the Virtex 4 family.  Virtex 5 FPGAs have 4 LUTs in a slice and are called “{A, B, C, D}{5, 6}LUT” which have the capability to act as a 5 input or 6 input LUT.

### LUT Equation Syntax

Xilinx uses the following syntax for operators in a LUT equations string:

| Operator | Operator Meaning |
| --- | --- |
| * | Logical AND |
| + | Logical OR |
| @ | Logical XOR |
| ~ | Unary NOT |

The parenthesis characters are also used to denote precedence in equation. Valid equation values are A1, A2, A3 and A4 for 4-input LUTs with an additional A5 for 5-input LUTs and A6 for 6-input LUTs.  Some examples of LUT equations are:

| LUT Equation | Equation Meaning |
| --- | --- |
| A1 | Passes through the signal A1 |
| A1*A2 | A 2-input AND gate using A1 and A2 |
| ~A4 | Inverts the signal on A4 |
| (A4+(A1+(A2+A3))) | A multi-level OR of 4 inputs |

### XDL LUT Equation Syntax

An instance is configured by zero or more attributes in a list where an attribute has the pattern

<Physical Name>:<Optional Logical Name>:<Value>.

The physical name of a LUT has been mentioned at the beginning of this subsection as being F or G for Virtex 4 parts and “{A, B, C, D}{5, 6}LUT” for Virtex 5 parts.  The optional logical name is generally chosen by the synthesis tools or whatever name propagated through to Xilinx NGDBuild and Map when the signal was converted.  It is only for reference back to the user’s original design and should correspond with the output of the LUT.  The value has the following syntax:

#LUT:D=<equation>

This syntax can also be found in the XDLRC primitive_defs.  If we put this altogether with our examples above we would get:

F:mySignal0:#LUT:D=A1

G:mySignal1:#LUT:D=A1*A2

F:mySignal2:#LUT:D=~A4

G:mySignal3:#LUT:D=(A4+(A1+(A2+A3)))

Notice that the value also contains the colon (‘:’).  RapidSmith defines the separation of the three components of an attribute as the first and second colons, any colons found after the second colon is part of the attribute value.

It is also of some value to recognize that the router can re-arrange LUT inputs to make routing easier and this can change equation to some extent.

## Appendix B: Hard Macros in XDL and RapidSmith (deprecated)

A hard macro is a collection of primitive instances and nets that have been placed and possibly routed.  The XDL construct that represents a hard macro is the “module”.  A hard macro file only has a single module and no other nets, instances or module instances present in the file.  The design name is "__XILINX_NMC_MACRO" which is a static variable in the Design class:

public static final String hardMacroDesignName = "__XILINX_NMC_MACRO";

### Xilinx NMC files

The NCD file format is not used for hard macros; instead, Xilinx uses the NMC format which is fairly similar.  NMC files can be converted to and from XDL using the same xdl executable. For example:

xdl –ncd2ndl myHardMacro.nmc [optional: myHardMacro.xdl]

xdl –xdl2ncd myHardMacro.xdl myHardMacro.nmc

The user must specify the output file name for NMC files when converting from XDL to NMC, otherwise it will output a file with the same name but NCD extension and the Xilinx tools won’t read it properly.  You may also need to apply the –force option if there is an error.

### Xilinx Hard Macros

There are several hidden restrictions and conventions that must be adhered to in order to create a valid Xilinx hard macro.  Here is a list of the known quirks to creating Xilinx-compatible hard macros:

- Any nets that are designated as GND or VCC are invalid (this will cause xdl to run in an infinite loop)

- The TIEOFF primitive found in INT tiles (switch matrix tiles) cannot be used in hard macros (this will cause a problem later on with map and/or par)

- Conventional “XDL_DUMMY” SLICEs which contain a “_NO_USER_LOGIC” attribute will not work with the Xilinx tools.

- Nets can only have 1 (one) hard macro port assigned to its pin list.

### RapidSmith Hard Macro Generator

RapidSmith provides a generic hard macro generator that will take a placed and routed NCD design file (or XDL equivalent) and replace IOBs with module ports among other transformations to make it a valid and legal hard macro.  The hard macro generator provided is the same as used in the published paper, “Using Hard Macros to Reduce FPGA Compilation Time”, C. Lavin, et al., FPL’2010.  This class is found in the util package and can be run from the command line:

java edu.byu.ece.rapidSmith.util.HardMacroGenerator <input.xdl | input.ncd> <output file type: xdl | nmc> [optional: original_toplevel.vhd]

The hard macro generator has to remove TIEOFFs (which supply GND and VCC) because they are not legal primitives in hard macros.  Therefore, the hard macro generator creates input ports accordingly.  It also renames ports according to primitive instance names of the IOBs and will attempt to name them as they were found in the original HDL.  However, it only can guess, and for a more complete naming, the top level VHDL file can optionally be provided as a parameter to help the generator get the port names correctly.  Unfortunately, the generator cannot parse Verilog.

## Appendix C: Xilinx Family Names and Part Names

The part and family naming conventions used in RapidSmith largely follow those used in the Xilinx tool partgen.  RapidSmith includes an enum type called FamilyType for all known family architectures in the util package.

### Xilinx Part Names in RapidSmith

RapidSmith uses the part name pattern as produced by the Xilinx partgen tool.  These part names start with ‘X’ for Xilinx and are then often followed by a ‘C’ for commercial parts, ‘A’ for automotive parts, ‘Q’ for military grade parts, and ‘QR’ for space grade parts.  The part names also include the package, however, because RapidSmith does not have any timing information, the speed grade is optional.  Some examples are shown below:

| Examples of valid part names in RapidSmith | Examples of invalid part names in RapidSmith |
| --- | --- |
| - XC4VFX12FF668 - XC5LX110TFF1136-2 - XCV50BG256 | - Virtex 4 LX30 - XC5VSX35T-2FF665C - XC5VLX20T |

RapidSmith contains methods in util.RunXilinxTools to automatically run partgen and parse its output to obtain part names of installed devices.  The installer uses these methods in order to determine which parts are valid on the system.  The user can also refer to the final table in the Appendix entry on Memory and Performance of RapidSmith for a long list of part names.  Other manipulation and conversion function for part names are found in the util.PartNameTools class.

### Xilinx Family Names in RapidSmith

By using the FamilyType enum, it makes writing code easier when trying to figure out what family the current design is targeting.  The util.PartNameTools has several methods to help identify a family type from a part name and also identify sub family names.  With the most recent Xilinx tools (ISE 11.1 and above) do not support legacy devices (Spartan 2, Spartan 2E, Virtex, Virtex E, Virtex 2 and Virtex 2 Pro families) and must use ISE 10.1.03 or older to create XDLRC reports and import/export XDL design.  PartNameTools contains a method to determine if a given family type is a legacy type:

/**

* This method determines which family types require the older version

* of the Xilinx tools (10.1.03 or older).

* @param familyType The family type to check.

* @return True if this part requires older tools (10.1.03) or older, false

* otherwise.

*/

public static boolean isFamilyTypeLegacy(FamilyType familyType);

## Appendix D: XDLRC Compatible Families (deprecated)

RapidSmith depends on Xilinx XDLRC files for device descriptions.  Xilinx offers several different families which in actuality use the same XDLRC information.  Therefore, there is a set of base (commercial grade) families that are compatible with other families as well.  The follow is a table showing these compatibilities:

| Base Families (Formal Name) | Base Families (partgen Name) | XDLRC Compatible Families |
| --- | --- | --- |
| Spartan2 | spartan2 | - |
| Spartan2E | spartan2e | aspartan2e |
| Spartan3 | spartan3 | aspartan3 |
| Spartan3A and Spartan3AN | spartan3a | aspartan3a |
| Spartan-3A DSP | spartan3adsp | aspartan3adsp |
| Spartan3E | spartan3e | aspartan3e |
| Spartan6 | spartan6 | spartan6l, aspartan6, qspartan6, qspartan6l |
| Virtex | virtex | qvirtex, qrvirtex |
| Virtex2 | virtex2 | qvirtex2, qrvirtex2 |
| Virtex2P | virtex2p | qvirtex2p |
| Virtex4 | virtex4 | qrvirtex4, qvirtex4 |
| Virtex5 | virtex5 | qvirtex5, qrvirtex5 |
| Virtex6 | virtex6 | virtex6l |
| VirtexE | virtexe | qvirtexe |

Only device files are generated for the base families.  Therefore, when loading designs or RapidSmith files, it’s always best to use the base family name.  One way to ensure this is to use the following method:

/**

* This method will take a familyType and return the base familyType

* architecture.  For example, the XDLRC RapidSmith uses for Automotive

* Spartan 6, Low Power Spartan 6 and Military Grade Spartan 6 all have

* the same base architecture: Spartan 6.  This method determines the

* base architecture based on the familyType.

* @param type The given family type.

* @return The base family type architecture.

*/

public static FamilyType getBaseTypeFromFamilyType(FamilyType type);

## Appendix E: Memory and Performance of RapidSmith (deprecated)

This section contains a dump from the shell script generated in the tests.TestFileLoading class in RapidSmith.  It determines the files size, Java heap usage in MB and load time from disk for the following 3 types of files. This script was run on a Windows 7 Enterprise 64-bit OS with an Intel Core i7 860 running at 2.8GHz.  The test machine had 8 GB of DDR3 memory, a conventional 1 TB SATA hard drive and was using version 0.5.0 of RapidSmith and the 32-bit Oracle JVM version 1.6.0_22.

Running RapidSmith in 64-bit Java will work exactly the same as in 32-bit mode; however its performance will be slightly slower and use more memory.

### Wire Enumerator Files

| Family Name | File Size | Java Heap Usage | Load Time From Disk |
| --- | --- | --- | --- |
| SPARTAN2 | 17KB | 0.6MB | 0.030s |
| SPARTAN2E | 19KB | 0.6MB | 0.030s |
| SPARTAN3 | 15KB | 0.5MB | 0.030s |
| SPARTAN3A | 25KB | 0.9MB | 0.032s |
| SPARTAN3ADSP | 33KB | 1.2MB | 0.035s |
| SPARTAN3E | 26KB | 0.9MB | 0.033s |
| SPARTAN6 | 168KB | 6.6MB | 0.081s |
| VIRTEX | 17KB | 0.6MB | 0.030s |
| VIRTEX2 | 21KB | 0.7MB | 0.033s |
| VIRTEX2P | 90KB | 3.1MB | 0.055s |
| VIRTEX4 | 233KB | 8.1MB | 0.109s |
| VIRTEX5 | 264KB | 9.9MB | 0.128s |
| VIRTEX6 | 171KB | 6.3MB | 0.098s |

### Primitive Definitions Files

| Family Name | File Size | Java Heap Usage | Load Time From Disk |
| --- | --- | --- | --- |
| SPARTAN2 | 8KB | 0.1MB | 0.085s |
| SPARTAN2E | 9KB | 0.1MB | 0.084s |
| SPARTAN3 | 17KB | 0.1MB | 0.082s |
| SPARTAN3A | 23KB | 0.1MB | 0.097s |
| SPARTAN3ADSP | 63KB | 0.1MB | 0.110s |
| SPARTAN3E | 21KB | 0.1MB | 0.085s |
| SPARTAN6 | 117KB | 0.1MB | 0.114s |
| VIRTEX | 8KB | 0.1MB | 0.075s |
| VIRTEX2 | 18KB | 0.1MB | 0.079s |
| VIRTEX2P | 88KB | 0.1MB | 0.108s |
| VIRTEX4 | 179KB | 0.1MB | 0.144s |
| VIRTEX5 | 322KB | 0.1MB | 0.191s |
| VIRTEX6 | 191KB | 0.1MB | 0.149s |

### Device Files

| Family Name | Part Name | File Size | Java Heap Size | Load Time from Disk |
| --- | --- | --- | --- | --- |
| KINTEX7 | xc7k160tfbg484 | 1093KB | 40MB | 0.430s |
| KINTEX7 | xc7k160tfbg676 | 1093KB | 40MB | 0.424s |
| KINTEX7 | xc7k160tffg676 | 1093KB | 40MB | 0.442s |
| KINTEX7 | xc7k30tfbg484 | 619KB | 17MB | 0.221s |
| KINTEX7 | xc7k30tsbg324 | 619KB | 17MB | 0.209s |
| KINTEX7 | xc7k325tfbg676 | 1424KB | 58MB | 0.628s |
| KINTEX7 | xc7k325tfbg900 | 1424KB | 58MB | 0.628s |
| KINTEX7 | xc7k325tffg676 | 1424KB | 58MB | 0.629s |
| KINTEX7 | xc7k325tffg900 | 1424KB | 58MB | 0.665s |
| KINTEX7 | xc7k410tfbg676 | 1704KB | 72MB | 0.774s |
| KINTEX7 | xc7k410tfbg900 | 1705KB | 72MB | 0.777s |
| KINTEX7 | xc7k410tffg676 | 1704KB | 72MB | 0.807s |
| KINTEX7 | xc7k410tffg900 | 1705KB | 72MB | 0.797s |
| KINTEX7 | xc7k70tfbg484 | 778KB | 25MB | 0.345s |
| KINTEX7 | xc7k70tfbg676 | 778KB | 25MB | 0.358s |
| KINTEX7 | xc7k70tsbg324 | 778KB | 25MB | 0.324s |
| SPARTAN2 | xc2s100fg256 | 134KB | 3MB | 0.082s |
| SPARTAN2 | xc2s100fg456 | 134KB | 3MB | 0.082s |
| SPARTAN2 | xc2s100pq208 | 134KB | 3MB | 0.083s |
| SPARTAN2 | xc2s100tq144 | 134KB | 3MB | 0.091s |
| SPARTAN2 | xc2s150fg256 | 150KB | 5MB | 0.096s |
| SPARTAN2 | xc2s150fg456 | 150KB | 5MB | 0.101s |
| SPARTAN2 | xc2s150pq208 | 150KB | 5MB | 0.096s |
| SPARTAN2 | xc2s15cs144 | 84KB | 1MB | 0.054s |
| SPARTAN2 | xc2s15tq144 | 84KB | 1MB | 0.052s |
| SPARTAN2 | xc2s15vq100 | 84KB | 1MB | 0.057s |
| SPARTAN2 | xc2s200fg256 | 179KB | 7MB | 0.115s |
| SPARTAN2 | xc2s200fg456 | 179KB | 7MB | 0.119s |
| SPARTAN2 | xc2s200pq208 | 179KB | 7MB | 0.116s |
| SPARTAN2 | xc2s30cs144 | 103KB | 1MB | 0.062s |
| SPARTAN2 | xc2s30pq208 | 103KB | 1MB | 0.059s |
| SPARTAN2 | xc2s30tq144 | 103KB | 1MB | 0.060s |
| SPARTAN2 | xc2s30vq100 | 103KB | 1MB | 0.060s |
| SPARTAN2 | xc2s50fg256 | 117KB | 2MB | 0.078s |
| SPARTAN2 | xc2s50pq208 | 117KB | 2MB | 0.078s |
| SPARTAN2 | xc2s50tq144 | 118KB | 2MB | 0.082s |
| SPARTAN2E | xc2s100efg456 | 137KB | 3MB | 0.102s |
| SPARTAN2E | xc2s100eft256 | 137KB | 3MB | 0.095s |
| SPARTAN2E | xc2s100epq208 | 137KB | 3MB | 0.095s |
| SPARTAN2E | xc2s100etq144 | 137KB | 3MB | 0.086s |
| SPARTAN2E | xc2s150efg456 | 154KB | 5MB | 0.095s |
| SPARTAN2E | xc2s150eft256 | 154KB | 5MB | 0.097s |
| SPARTAN2E | xc2s150epq208 | 154KB | 5MB | 0.101s |
| SPARTAN2E | xc2s200efg456 | 189KB | 7MB | 0.119s |
| SPARTAN2E | xc2s200eft256 | 189KB | 7MB | 0.118s |
| SPARTAN2E | xc2s200epq208 | 189KB | 7MB | 0.121s |
| SPARTAN2E | xc2s300efg456 | 233KB | 8MB | 0.129s |
| SPARTAN2E | xc2s300eft256 | 233KB | 8MB | 0.129s |
| SPARTAN2E | xc2s300epq208 | 233KB | 8MB | 0.128s |
| SPARTAN2E | xc2s400efg456 | 326KB | 13MB | 0.182s |
| SPARTAN2E | xc2s400efg676 | 326KB | 13MB | 0.179s |
| SPARTAN2E | xc2s400eft256 | 326KB | 13MB | 0.180s |
| SPARTAN2E | xc2s50eft256 | 121KB | 2MB | 0.076s |
| SPARTAN2E | xc2s50epq208 | 121KB | 2MB | 0.076s |
| SPARTAN2E | xc2s50etq144 | 121KB | 2MB | 0.076s |
| SPARTAN2E | xc2s600efg456 | 390KB | 19MB | 0.235s |
| SPARTAN2E | xc2s600efg676 | 389KB | 19MB | 0.259s |
| SPARTAN3 | xc3s1000fg320 | 357KB | 19MB | 0.199s |
| SPARTAN3 | xc3s1000fg456 | 357KB | 19MB | 0.187s |
| SPARTAN3 | xc3s1000fg676 | 357KB | 19MB | 0.186s |
| SPARTAN3 | xc3s1000ft256 | 357KB | 19MB | 0.186s |
| SPARTAN3 | xc3s1000lfg320 | 357KB | 19MB | 0.185s |
| SPARTAN3 | xc3s1000lfg456 | 357KB | 19MB | 0.185s |
| SPARTAN3 | xc3s1000lft256 | 357KB | 19MB | 0.185s |
| SPARTAN3 | xc3s1500fg320 | 538KB | 31MB | 0.270s |
| SPARTAN3 | xc3s1500fg456 | 538KB | 31MB | 0.271s |
| SPARTAN3 | xc3s1500fg676 | 538KB | 31MB | 0.286s |
| SPARTAN3 | xc3s1500lfg320 | 538KB | 31MB | 0.284s |
| SPARTAN3 | xc3s1500lfg456 | 538KB | 31MB | 0.260s |
| SPARTAN3 | xc3s1500lfg676 | 538KB | 31MB | 0.260s |
| SPARTAN3 | xc3s2000fg456 | 739KB | 48MB | 0.350s |
| SPARTAN3 | xc3s2000fg676 | 739KB | 48MB | 0.354s |
| SPARTAN3 | xc3s2000fg900 | 739KB | 48MB | 0.359s |
| SPARTAN3 | xc3s200ft256 | 106KB | 5MB | 0.081s |
| SPARTAN3 | xc3s200pq208 | 106KB | 5MB | 0.083s |
| SPARTAN3 | xc3s200tq144 | 106KB | 5MB | 0.086s |
| SPARTAN3 | xc3s200vq100 | 106KB | 5MB | 0.084s |
| SPARTAN3 | xc3s4000fg1156 | 912KB | 66MB | 0.508s |
| SPARTAN3 | xc3s4000fg676 | 912KB | 66MB | 0.513s |
| SPARTAN3 | xc3s4000fg900 | 912KB | 66MB | 0.518s |
| SPARTAN3 | xc3s4000lfg900 | 912KB | 66MB | 0.509s |
| SPARTAN3 | xc3s400fg320 | 203KB | 9MB | 0.145s |
| SPARTAN3 | xc3s400fg456 | 203KB | 9MB | 0.174s |
| SPARTAN3 | xc3s400ft256 | 203KB | 9MB | 0.203s |
| SPARTAN3 | xc3s400pq208 | 203KB | 9MB | 0.220s |
| SPARTAN3 | xc3s400tq144 | 203KB | 9MB | 0.190s |
| SPARTAN3 | xc3s5000fg1156 | 1184KB | 79MB | 0.707s |
| SPARTAN3 | xc3s5000fg676 | 1184KB | 79MB | 0.612s |
| SPARTAN3 | xc3s5000fg900 | 1184KB | 79MB | 0.611s |
| SPARTAN3 | xc3s50cp132 | 78KB | 2MB | 0.060s |
| SPARTAN3 | xc3s50pq208 | 78KB | 2MB | 0.060s |
| SPARTAN3 | xc3s50tq144 | 78KB | 2MB | 0.060s |
| SPARTAN3 | xc3s50vq100 | 78KB | 2MB | 0.061s |
| SPARTAN3A | xc3s1400afg484 | 630KB | 28MB | 0.274s |
| SPARTAN3A | xc3s1400afg676 | 630KB | 28MB | 0.284s |
| SPARTAN3A | xc3s1400aft256 | 630KB | 28MB | 0.291s |
| SPARTAN3A | xc3s1400anfgg676 | 630KB | 28MB | 0.271s |
| SPARTAN3A | xc3s200afg320 | 194KB | 6MB | 0.098s |
| SPARTAN3A | xc3s200aft256 | 194KB | 6MB | 0.096s |
| SPARTAN3A | xc3s200anftg256 | 194KB | 6MB | 0.096s |
| SPARTAN3A | xc3s200avq100 | 194KB | 6MB | 0.097s |
| SPARTAN3A | xc3s400afg320 | 309KB | 10MB | 0.121s |
| SPARTAN3A | xc3s400afg400 | 309KB | 10MB | 0.120s |
| SPARTAN3A | xc3s400aft256 | 309KB | 10MB | 0.119s |
| SPARTAN3A | xc3s400anfgg400 | 309KB | 10MB | 0.121s |
| SPARTAN3A | xc3s50aft256 | 114KB | 2MB | 0.070s |
| SPARTAN3A | xc3s50antqg144 | 114KB | 2MB | 0.073s |
| SPARTAN3A | xc3s50atq144 | 114KB | 2MB | 0.077s |
| SPARTAN3A | xc3s50avq100 | 114KB | 2MB | 0.081s |
| SPARTAN3A | xc3s700afg400 | 456KB | 16MB | 0.174s |
| SPARTAN3A | xc3s700afg484 | 456KB | 16MB | 0.163s |
| SPARTAN3A | xc3s700aft256 | 456KB | 16MB | 0.163s |
| SPARTAN3A | xc3s700anfgg484 | 456KB | 16MB | 0.171s |
| SPARTAN3ADSP | xc3sd1800acs484 | 1073KB | 45MB | 0.385s |
| SPARTAN3ADSP | xc3sd1800afg676 | 1073KB | 45MB | 0.349s |
| SPARTAN3ADSP | xc3sd3400acs484 | 1443KB | 65MB | 0.490s |
| SPARTAN3ADSP | xc3sd3400afg676 | 1443KB | 65MB | 0.496s |
| SPARTAN3E | xc3s100ecp132 | 152KB | 3MB | 0.075s |
| SPARTAN3E | xc3s100etq144 | 152KB | 3MB | 0.075s |
| SPARTAN3E | xc3s100evq100 | 152KB | 3MB | 0.075s |
| SPARTAN3E | xc3s1200efg320 | 588KB | 22MB | 0.213s |
| SPARTAN3E | xc3s1200efg400 | 588KB | 22MB | 0.220s |
| SPARTAN3E | xc3s1200eft256 | 588KB | 22MB | 0.217s |
| SPARTAN3E | xc3s1600efg320 | 794KB | 36MB | 0.299s |
| SPARTAN3E | xc3s1600efg400 | 794KB | 36MB | 0.302s |
| SPARTAN3E | xc3s1600efg484 | 794KB | 36MB | 0.300s |
| SPARTAN3E | xc3s250ecp132 | 234KB | 7MB | 0.102s |
| SPARTAN3E | xc3s250eft256 | 234KB | 7MB | 0.102s |
| SPARTAN3E | xc3s250epq208 | 234KB | 7MB | 0.102s |
| SPARTAN3E | xc3s250etq144 | 234KB | 7MB | 0.104s |
| SPARTAN3E | xc3s250evq100 | 234KB | 7MB | 0.102s |
| SPARTAN3E | xc3s500ecp132 | 376KB | 13MB | 0.159s |
| SPARTAN3E | xc3s500efg320 | 376KB | 13MB | 0.161s |
| SPARTAN3E | xc3s500eft256 | 376KB | 13MB | 0.162s |
| SPARTAN3E | xc3s500epq208 | 376KB | 13MB | 0.155s |
| SPARTAN3E | xc3s500evq100 | 376KB | 13MB | 0.151s |
| SPARTAN6 | xc6slx100csg484 | 530KB | 16MB | 0.221s |
| SPARTAN6 | xc6slx100fgg484 | 530KB | 16MB | 0.220s |
| SPARTAN6 | xc6slx100fgg676 | 530KB | 16MB | 0.221s |
| SPARTAN6 | xc6slx100tcsg484 | 620KB | 16MB | 0.222s |
| SPARTAN6 | xc6slx100tfgg484 | 620KB | 16MB | 0.222s |
| SPARTAN6 | xc6slx100tfgg676 | 620KB | 16MB | 0.224s |
| SPARTAN6 | xc6slx100tfgg900 | 620KB | 16MB | 0.234s |
| SPARTAN6 | xc6slx150csg484 | 624KB | 19MB | 0.262s |
| SPARTAN6 | xc6slx150fgg484 | 624KB | 19MB | 0.249s |
| SPARTAN6 | xc6slx150fgg676 | 624KB | 19MB | 0.245s |
| SPARTAN6 | xc6slx150fgg900 | 624KB | 19MB | 0.247s |
| SPARTAN6 | xc6slx150tcsg484 | 713KB | 20MB | 0.244s |
| SPARTAN6 | xc6slx150tfgg484 | 713KB | 20MB | 0.245s |
| SPARTAN6 | xc6slx150tfgg676 | 713KB | 20MB | 0.244s |
| SPARTAN6 | xc6slx150tfgg900 | 713KB | 20MB | 0.251s |
| SPARTAN6 | xc6slx16cpg196 | 234KB | 5MB | 0.106s |
| SPARTAN6 | xc6slx16csg225 | 234KB | 5MB | 0.106s |
| SPARTAN6 | xc6slx16csg324 | 234KB | 5MB | 0.117s |
| SPARTAN6 | xc6slx16ftg256 | 234KB | 5MB | 0.100s |
| SPARTAN6 | xc6slx25csg324 | 307KB | 8MB | 0.124s |
| SPARTAN6 | xc6slx25fgg484 | 307KB | 8MB | 0.123s |
| SPARTAN6 | xc6slx25ftg256 | 307KB | 8MB | 0.125s |
| SPARTAN6 | xc6slx25tcsg324 | 362KB | 8MB | 0.143s |
| SPARTAN6 | xc6slx25tfgg484 | 362KB | 8MB | 0.148s |
| SPARTAN6 | xc6slx45csg324 | 359KB | 9MB | 0.151s |
| SPARTAN6 | xc6slx45csg484 | 359KB | 9MB | 0.151s |
| SPARTAN6 | xc6slx45fgg484 | 359KB | 9MB | 0.145s |
| SPARTAN6 | xc6slx45fgg676 | 359KB | 9MB | 0.143s |
| SPARTAN6 | xc6slx45tcsg324 | 421KB | 10MB | 0.146s |
| SPARTAN6 | xc6slx45tcsg484 | 421KB | 10MB | 0.144s |
| SPARTAN6 | xc6slx45tfgg484 | 421KB | 10MB | 0.144s |
| SPARTAN6 | xc6slx4cpg196 | 150KB | 3MB | 0.083s |
| SPARTAN6 | xc6slx4csg225 | 150KB | 3MB | 0.085s |
| SPARTAN6 | xc6slx4tqg144 | 150KB | 3MB | 0.082s |
| SPARTAN6 | xc6slx75csg484 | 468KB | 13MB | 0.192s |
| SPARTAN6 | xc6slx75fgg484 | 468KB | 13MB | 0.202s |
| SPARTAN6 | xc6slx75fgg676 | 468KB | 13MB | 0.209s |
| SPARTAN6 | xc6slx75tcsg484 | 555KB | 13MB | 0.218s |
| SPARTAN6 | xc6slx75tfgg484 | 555KB | 13MB | 0.202s |
| SPARTAN6 | xc6slx75tfgg676 | 555KB | 13MB | 0.201s |
| SPARTAN6 | xc6slx9cpg196 | 215KB | 4MB | 0.089s |
| SPARTAN6 | xc6slx9csg225 | 215KB | 4MB | 0.089s |
| SPARTAN6 | xc6slx9csg324 | 215KB | 4MB | 0.088s |
| SPARTAN6 | xc6slx9ftg256 | 215KB | 4MB | 0.089s |
| SPARTAN6 | xc6slx9tqg144 | 215KB | 4MB | 0.088s |
| VIRTEX | xcv1000bg560 | 465KB | 31MB | 0.338s |
| VIRTEX | xcv1000fg680 | 465KB | 31MB | 0.346s |
| VIRTEX | xcv100bg256 | 132KB | 3MB | 0.087s |
| VIRTEX | xcv100cs144 | 132KB | 3MB | 0.088s |
| VIRTEX | xcv100fg256 | 132KB | 3MB | 0.092s |
| VIRTEX | xcv100pq240 | 132KB | 3MB | 0.083s |
| VIRTEX | xcv100tq144 | 132KB | 3MB | 0.082s |
| VIRTEX | xcv150bg256 | 148KB | 5MB | 0.095s |
| VIRTEX | xcv150bg352 | 148KB | 5MB | 0.103s |
| VIRTEX | xcv150fg256 | 148KB | 5MB | 0.102s |
| VIRTEX | xcv150fg456 | 148KB | 5MB | 0.103s |
| VIRTEX | xcv150pq240 | 148KB | 5MB | 0.099s |
| VIRTEX | xcv200bg256 | 176KB | 7MB | 0.110s |
| VIRTEX | xcv200bg352 | 177KB | 7MB | 0.109s |
| VIRTEX | xcv200fg256 | 176KB | 7MB | 0.114s |
| VIRTEX | xcv200fg456 | 177KB | 7MB | 0.116s |
| VIRTEX | xcv200pq240 | 176KB | 7MB | 0.117s |
| VIRTEX | xcv300bg352 | 216KB | 8MB | 0.131s |
| VIRTEX | xcv300bg432 | 216KB | 8MB | 0.125s |
| VIRTEX | xcv300fg456 | 216KB | 8MB | 0.123s |
| VIRTEX | xcv300pq240 | 216KB | 8MB | 0.125s |
| VIRTEX | xcv400bg432 | 274KB | 13MB | 0.173s |
| VIRTEX | xcv400bg560 | 274KB | 13MB | 0.183s |
| VIRTEX | xcv400fg676 | 274KB | 13MB | 0.183s |
| VIRTEX | xcv400hq240 | 274KB | 13MB | 0.181s |
| VIRTEX | xcv50bg256 | 116KB | 2MB | 0.077s |
| VIRTEX | xcv50cs144 | 116KB | 2MB | 0.075s |
| VIRTEX | xcv50fg256 | 116KB | 2MB | 0.075s |
| VIRTEX | xcv50pq240 | 116KB | 2MB | 0.076s |
| VIRTEX | xcv50tq144 | 116KB | 2MB | 0.076s |
| VIRTEX | xcv600bg432 | 330KB | 18MB | 0.229s |
| VIRTEX | xcv600bg560 | 330KB | 18MB | 0.228s |
| VIRTEX | xcv600fg676 | 330KB | 18MB | 0.228s |
| VIRTEX | xcv600fg680 | 330KB | 18MB | 0.229s |
| VIRTEX | xcv600hq240 | 330KB | 18MB | 0.243s |
| VIRTEX | xcv800bg432 | 399KB | 24MB | 0.322s |
| VIRTEX | xcv800bg560 | 399KB | 24MB | 0.308s |
| VIRTEX | xcv800fg676 | 399KB | 24MB | 0.317s |
| VIRTEX | xcv800fg680 | 399KB | 24MB | 0.314s |
| VIRTEX | xcv800hq240 | 399KB | 24MB | 0.311s |
| VIRTEX2 | xc2v1000bg575 | 378KB | 15MB | 0.168s |
| VIRTEX2 | xc2v1000ff896 | 378KB | 15MB | 0.168s |
| VIRTEX2 | xc2v1000fg256 | 378KB | 15MB | 0.170s |
| VIRTEX2 | xc2v1000fg456 | 378KB | 15MB | 0.186s |
| VIRTEX2 | xc2v1500bg575 | 496KB | 21MB | 0.230s |
| VIRTEX2 | xc2v1500ff896 | 496KB | 21MB | 0.223s |
| VIRTEX2 | xc2v1500fg676 | 496KB | 21MB | 0.218s |
| VIRTEX2 | xc2v2000bf957 | 599KB | 28MB | 0.288s |
| VIRTEX2 | xc2v2000bg575 | 599KB | 28MB | 0.287s |
| VIRTEX2 | xc2v2000ff896 | 599KB | 28MB | 0.287s |
| VIRTEX2 | xc2v2000fg676 | 599KB | 28MB | 0.288s |
| VIRTEX2 | xc2v250cs144 | 171KB | 6MB | 0.099s |
| VIRTEX2 | xc2v250fg256 | 171KB | 6MB | 0.104s |
| VIRTEX2 | xc2v250fg456 | 171KB | 6MB | 0.106s |
| VIRTEX2 | xc2v3000bf957 | 759KB | 38MB | 0.410s |
| VIRTEX2 | xc2v3000bg728 | 759KB | 38MB | 0.374s |
| VIRTEX2 | xc2v3000ff1152 | 759KB | 38MB | 0.374s |
| VIRTEX2 | xc2v3000fg676 | 759KB | 38MB | 0.377s |
| VIRTEX2 | xc2v4000bf957 | 1083KB | 60MB | 0.548s |
| VIRTEX2 | xc2v4000ff1152 | 1083KB | 60MB | 0.593s |
| VIRTEX2 | xc2v4000ff1517 | 1083KB | 60MB | 0.565s |
| VIRTEX2 | xc2v40cs144 | 96KB | 1MB | 0.057s |
| VIRTEX2 | xc2v40fg256 | 96KB | 1MB | 0.060s |
| VIRTEX2 | xc2v500fg256 | 290KB | 10MB | 0.139s |
| VIRTEX2 | xc2v500fg456 | 290KB | 10MB | 0.132s |
| VIRTEX2 | xc2v6000bf957 | 1381KB | 86MB | 0.750s |
| VIRTEX2 | xc2v6000ff1152 | 1381KB | 86MB | 0.790s |
| VIRTEX2 | xc2v6000ff1517 | 1381KB | 86MB | 0.772s |
| VIRTEX2 | xc2v8000ff1152 | 2205KB | 117MB | 1.070s |
| VIRTEX2 | xc2v8000ff1517 | 2205KB | 117MB | 1.024s |
| VIRTEX2 | xc2v80cs144 | 113KB | 2MB | 0.070s |
| VIRTEX2 | xc2v80fg256 | 113KB | 2MB | 0.070s |
| VIRTEX2P | xc2vp100ff1696 | 2520KB | 125MB | 0.987s |
| VIRTEX2P | xc2vp100ff1704 | 2515KB | 125MB | 1.007s |
| VIRTEX2P | xc2vp20ff1152 | 869KB | 29MB | 0.292s |
| VIRTEX2P | xc2vp20ff896 | 869KB | 29MB | 0.281s |
| VIRTEX2P | xc2vp20fg676 | 868KB | 29MB | 0.277s |
| VIRTEX2P | xc2vp2ff672 | 214KB | 5MB | 0.090s |
| VIRTEX2P | xc2vp2fg256 | 214KB | 5MB | 0.096s |
| VIRTEX2P | xc2vp2fg456 | 214KB | 5MB | 0.097s |
| VIRTEX2P | xc2vp30ff1152 | 1105KB | 41MB | 0.380s |
| VIRTEX2P | xc2vp30ff896 | 1105KB | 41MB | 0.365s |
| VIRTEX2P | xc2vp30fg676 | 1105KB | 41MB | 0.399s |
| VIRTEX2P | xc2vp40ff1148 | 1407KB | 57MB | 0.530s |
| VIRTEX2P | xc2vp40ff1152 | 1407KB | 57MB | 0.502s |
| VIRTEX2P | xc2vp40fg676 | 1407KB | 57MB | 0.517s |
| VIRTEX2P | xc2vp4ff672 | 450KB | 11MB | 0.139s |
| VIRTEX2P | xc2vp4fg256 | 450KB | 11MB | 0.128s |
| VIRTEX2P | xc2vp4fg456 | 450KB | 11MB | 0.127s |
| VIRTEX2P | xc2vp50ff1148 | 1601KB | 70MB | 0.610s |
| VIRTEX2P | xc2vp50ff1152 | 1596KB | 70MB | 0.564s |
| VIRTEX2P | xc2vp50ff1517 | 1595KB | 70MB | 0.565s |
| VIRTEX2P | xc2vp70ff1517 | 2052KB | 96MB | 0.769s |
| VIRTEX2P | xc2vp70ff1704 | 2052KB | 96MB | 0.713s |
| VIRTEX2P | xc2vp7ff672 | 610KB | 17MB | 0.173s |
| VIRTEX2P | xc2vp7ff896 | 610KB | 17MB | 0.174s |
| VIRTEX2P | xc2vp7fg456 | 610KB | 17MB | 0.174s |
| VIRTEX2P | xc2vpx20ff896 | 892KB | 30MB | 0.280s |
| VIRTEX2P | xc2vpx70ff1704 | 2117KB | 96MB | 0.788s |
| VIRTEX4 | xc4vfx100ff1152 | 1357KB | 57MB | 0.550s |
| VIRTEX4 | xc4vfx100ff1517 | 1357KB | 57MB | 0.579s |
| VIRTEX4 | xc4vfx12ff668 | 598KB | 18MB | 0.179s |
| VIRTEX4 | xc4vfx12sf363 | 598KB | 18MB | 0.179s |
| VIRTEX4 | xc4vfx140ff1517 | 1546KB | 70MB | 0.616s |
| VIRTEX4 | xc4vfx20ff672 | 924KB | 27MB | 0.297s |
| VIRTEX4 | xc4vfx40ff1152 | 1159KB | 41MB | 0.366s |
| VIRTEX4 | xc4vfx40ff672 | 1158KB | 41MB | 0.367s |
| VIRTEX4 | xc4vfx60ff1152 | 1206KB | 46MB | 0.386s |
| VIRTEX4 | xc4vfx60ff672 | 1206KB | 46MB | 0.385s |
| VIRTEX4 | xc4vlx100ff1148 | 701KB | 37MB | 0.384s |
| VIRTEX4 | xc4vlx100ff1513 | 701KB | 37MB | 0.405s |
| VIRTEX4 | xc4vlx15ff668 | 231KB | 16MB | 0.151s |
| VIRTEX4 | xc4vlx15ff676 | 231KB | 16MB | 0.152s |
| VIRTEX4 | xc4vlx15sf363 | 231KB | 16MB | 0.153s |
| VIRTEX4 | xc4vlx160ff1148 | 875KB | 55MB | 0.540s |
| VIRTEX4 | xc4vlx160ff1513 | 875KB | 55MB | 0.538s |
| VIRTEX4 | xc4vlx200ff1513 | 1010KB | 61MB | 0.602s |
| VIRTEX4 | xc4vlx25ff668 | 287KB | 18MB | 0.177s |
| VIRTEX4 | xc4vlx25ff676 | 287KB | 18MB | 0.170s |
| VIRTEX4 | xc4vlx25sf363 | 287KB | 18MB | 0.167s |
| VIRTEX4 | xc4vlx40ff1148 | 348KB | 21MB | 0.226s |
| VIRTEX4 | xc4vlx40ff668 | 349KB | 21MB | 0.222s |
| VIRTEX4 | xc4vlx60ff1148 | 464KB | 30MB | 0.292s |
| VIRTEX4 | xc4vlx60ff668 | 464KB | 30MB | 0.287s |
| VIRTEX4 | xc4vlx80ff1148 | 596KB | 35MB | 0.362s |
| VIRTEX4 | xc4vsx25ff668 | 373KB | 27MB | 0.275s |
| VIRTEX4 | xc4vsx35ff668 | 441KB | 28MB | 0.287s |
| VIRTEX4 | xc4vsx55ff1148 | 539KB | 34MB | 0.299s |
| VIRTEX5 | xc5vfx100tff1136 | 1014KB | 45MB | 0.448s |
| VIRTEX5 | xc5vfx100tff1738 | 1014KB | 45MB | 0.440s |
| VIRTEX5 | xc5vfx130tff1738 | 1058KB | 47MB | 0.522s |
| VIRTEX5 | xc5vfx200tff1738 | 1227KB | 60MB | 0.585s |
| VIRTEX5 | xc5vfx30tff665 | 781KB | 25MB | 0.306s |
| VIRTEX5 | xc5vfx70tff1136 | 899KB | 34MB | 0.350s |
| VIRTEX5 | xc5vfx70tff665 | 898KB | 34MB | 0.355s |
| VIRTEX5 | xc5vlx110ff1153 | 589KB | 32MB | 0.329s |
| VIRTEX5 | xc5vlx110ff1760 | 589KB | 32MB | 0.322s |
| VIRTEX5 | xc5vlx110ff676 | 588KB | 32MB | 0.321s |
| VIRTEX5 | xc5vlx110tff1136 | 761KB | 34MB | 0.341s |
| VIRTEX5 | xc5vlx110tff1738 | 761KB | 34MB | 0.340s |
| VIRTEX5 | xc5vlx155ff1153 | 716KB | 44MB | 0.423s |
| VIRTEX5 | xc5vlx155ff1760 | 716KB | 44MB | 0.436s |
| VIRTEX5 | xc5vlx155tff1136 | 893KB | 46MB | 0.427s |
| VIRTEX5 | xc5vlx155tff1738 | 893KB | 46MB | 0.426s |
| VIRTEX5 | xc5vlx20tff323 | 497KB | 17MB | 0.192s |
| VIRTEX5 | xc5vlx220ff1760 | 862KB | 62MB | 0.566s |
| VIRTEX5 | xc5vlx220tff1738 | 1039KB | 64MB | 0.619s |
| VIRTEX5 | xc5vlx30ff324 | 380KB | 18MB | 0.198s |
| VIRTEX5 | xc5vlx30ff676 | 381KB | 18MB | 0.183s |
| VIRTEX5 | xc5vlx30tff323 | 558KB | 20MB | 0.222s |
| VIRTEX5 | xc5vlx30tff665 | 558KB | 20MB | 0.221s |
| VIRTEX5 | xc5vlx330ff1760 | 1069KB | 69MB | 0.622s |
| VIRTEX5 | xc5vlx330tff1738 | 1250KB | 72MB | 0.632s |
| VIRTEX5 | xc5vlx50ff1153 | 417KB | 19MB | 0.231s |
| VIRTEX5 | xc5vlx50ff324 | 417KB | 19MB | 0.229s |
| VIRTEX5 | xc5vlx50ff676 | 417KB | 19MB | 0.217s |
| VIRTEX5 | xc5vlx50tff1136 | 587KB | 21MB | 0.235s |
| VIRTEX5 | xc5vlx50tff665 | 586KB | 21MB | 0.237s |
| VIRTEX5 | xc5vlx85ff1153 | 535KB | 30MB | 0.305s |
| VIRTEX5 | xc5vlx85ff676 | 534KB | 30MB | 0.303s |
| VIRTEX5 | xc5vlx85tff1136 | 706KB | 32MB | 0.321s |
| VIRTEX5 | xc5vsx240tff1738 | 1135KB | 61MB | 0.630s |
| VIRTEX5 | xc5vsx35tff665 | 596KB | 25MB | 0.257s |
| VIRTEX5 | xc5vsx50tff1136 | 630KB | 26MB | 0.300s |
| VIRTEX5 | xc5vsx50tff665 | 630KB | 26MB | 0.302s |
| VIRTEX5 | xc5vsx95tff1136 | 754KB | 36MB | 0.382s |
| VIRTEX5 | xc5vtx150tff1156 | 870KB | 40MB | 0.402s |
| VIRTEX5 | xc5vtx150tff1759 | 871KB | 40MB | 0.402s |
| VIRTEX5 | xc5vtx240tff1759 | 1111KB | 56MB | 0.620s |
| VIRTEX6 | xc6vcx130tff1156 | 715KB | 25MB | 0.333s |
| VIRTEX6 | xc6vcx130tff484 | 715KB | 25MB | 0.336s |
| VIRTEX6 | xc6vcx130tff784 | 715KB | 25MB | 0.332s |
| VIRTEX6 | xc6vcx195tff1156 | 853KB | 32MB | 0.418s |
| VIRTEX6 | xc6vcx195tff784 | 853KB | 32MB | 0.420s |
| VIRTEX6 | xc6vcx240tff1156 | 937KB | 35MB | 0.460s |
| VIRTEX6 | xc6vcx240tff784 | 937KB | 35MB | 0.447s |
| VIRTEX6 | xc6vcx75tff484 | 583KB | 20MB | 0.252s |
| VIRTEX6 | xc6vcx75tff784 | 584KB | 20MB | 0.251s |
| VIRTEX6 | xc6vhx250tff1154 | 973KB | 36MB | 0.444s |
| VIRTEX6 | xc6vhx255tff1155 | 1029KB | 38MB | 0.463s |
| VIRTEX6 | xc6vhx255tff1923 | 1029KB | 38MB | 0.496s |
| VIRTEX6 | xc6vhx380tff1154 | 1285KB | 47MB | 0.575s |
| VIRTEX6 | xc6vhx380tff1155 | 1286KB | 47MB | 0.563s |
| VIRTEX6 | xc6vhx380tff1923 | 1288KB | 47MB | 0.606s |
| VIRTEX6 | xc6vhx380tff1924 | 1287KB | 47MB | 0.583s |
| VIRTEX6 | xc6vhx565tff1923 | 1661KB | 66MB | 0.843s |
| VIRTEX6 | xc6vhx565tff1924 | 1660KB | 66MB | 0.874s |
| VIRTEX6 | xc6vlx130tff1156 | 709KB | 25MB | 0.361s |
| VIRTEX6 | xc6vlx130tff484 | 709KB | 25MB | 0.376s |
| VIRTEX6 | xc6vlx130tff784 | 709KB | 25MB | 0.366s |
| VIRTEX6 | xc6vlx195tff1156 | 849KB | 32MB | 0.433s |
| VIRTEX6 | xc6vlx195tff784 | 848KB | 32MB | 0.449s |
| VIRTEX6 | xc6vlx240tff1156 | 934KB | 35MB | 0.510s |
| VIRTEX6 | xc6vlx240tff1759 | 935KB | 35MB | 0.502s |
| VIRTEX6 | xc6vlx240tff784 | 934KB | 35MB | 0.459s |
| VIRTEX6 | xc6vlx365tff1156 | 1186KB | 46MB | 0.599s |
| VIRTEX6 | xc6vlx365tff1759 | 1187KB | 46MB | 0.599s |
| VIRTEX6 | xc6vlx550tff1759 | 1550KB | 62MB | 0.836s |
| VIRTEX6 | xc6vlx550tff1760 | 1551KB | 62MB | 0.814s |
| VIRTEX6 | xc6vlx75tff484 | 582KB | 20MB | 0.281s |
| VIRTEX6 | xc6vlx75tff784 | 582KB | 20MB | 0.283s |
| VIRTEX6 | xc6vlx760ff1760 | 1758KB | 77MB | 1.068s |
| VIRTEX6 | xc6vsx315tff1156 | 1157KB | 45MB | 0.562s |
| VIRTEX6 | xc6vsx315tff1759 | 1157KB | 45MB | 0.587s |
| VIRTEX6 | xc6vsx475tff1156 | 1505KB | 61MB | 0.814s |
| VIRTEX6 | xc6vsx475tff1759 | 1506KB | 61MB | 0.824s |
| VIRTEX7 | xc7v1500tffg1761 | 4985KB | 263MB | 2.653s |
| VIRTEX7 | xc7v1500tfhg1157 | 4985KB | 263MB | 2.709s |
| VIRTEX7 | xc7v2000tffg1925 | 5956KB | 301MB | 3.339s |
| VIRTEX7 | xc7v2000tfhg1761 | 5956KB | 301MB | 3.365s |
| VIRTEX7 | xc7v285tffg1157 | 1332KB | 53MB | 0.625s |
| VIRTEX7 | xc7v285tffg1761 | 1332KB | 53MB | 0.632s |
| VIRTEX7 | xc7v285tffg484 | 1331KB | 53MB | 0.625s |
| VIRTEX7 | xc7v285tffg784 | 1331KB | 53MB | 0.648s |
| VIRTEX7 | xc7v450tffg1157 | 1697KB | 71MB | 0.830s |
| VIRTEX7 | xc7v450tffg1761 | 1697KB | 71MB | 0.847s |
| VIRTEX7 | xc7v450tffg784 | 1697KB | 71MB | 0.838s |
| VIRTEX7 | xc7v585tffg1157 | 1996KB | 83MB | 1.004s |
| VIRTEX7 | xc7v585tffg1761 | 1996KB | 83MB | 0.968s |
| VIRTEX7 | xc7v855tffg1157 | 2634KB | 115MB | 1.408s |
| VIRTEX7 | xc7v855tffg1761 | 2634KB | 115MB | 1.461s |
| VIRTEX7 | xc7vx485tffg1157 | 2137KB | 93MB | 1.322s |
| VIRTEX7 | xc7vx485tffg1158 | 2136KB | 93MB | 1.083s |
| VIRTEX7 | xc7vx485tffg1761 | 2137KB | 93MB | 1.144s |
| VIRTEX7 | xc7vx485tffg1929 | 2136KB | 93MB | 1.124s |
| VIRTEXE | xcv1000ebg560 | 623KB | 31MB | 0.389s |
| VIRTEXE | xcv1000efg1156 | 623KB | 31MB | 0.388s |
| VIRTEXE | xcv1000efg680 | 623KB | 31MB | 0.402s |
| VIRTEXE | xcv1000efg860 | 623KB | 31MB | 0.393s |
| VIRTEXE | xcv1000efg900 | 623KB | 31MB | 0.391s |
| VIRTEXE | xcv1000ehq240 | 623KB | 31MB | 0.385s |
| VIRTEXE | xcv100ebg352 | 163KB | 4MB | 0.093s |
| VIRTEXE | xcv100ecs144 | 163KB | 4MB | 0.092s |
| VIRTEXE | xcv100efg256 | 163KB | 4MB | 0.089s |
| VIRTEXE | xcv100epq240 | 163KB | 4MB | 0.087s |
| VIRTEXE | xcv1600ebg560 | 778KB | 39MB | 0.490s |
| VIRTEXE | xcv1600efg1156 | 778KB | 39MB | 0.489s |
| VIRTEXE | xcv1600efg680 | 778KB | 39MB | 0.485s |
| VIRTEXE | xcv1600efg860 | 778KB | 39MB | 0.475s |
| VIRTEXE | xcv1600efg900 | 778KB | 39MB | 0.495s |
| VIRTEXE | xcv2000ebg560 | 860KB | 49MB | 0.640s |
| VIRTEXE | xcv2000efg1156 | 860KB | 49MB | 0.637s |
| VIRTEXE | xcv2000efg680 | 860KB | 49MB | 0.630s |
| VIRTEXE | xcv2000efg860 | 860KB | 49MB | 0.649s |
| VIRTEXE | xcv200ebg352 | 237KB | 7MB | 0.117s |
| VIRTEXE | xcv200ecs144 | 237KB | 7MB | 0.118s |
| VIRTEXE | xcv200efg256 | 237KB | 7MB | 0.117s |
| VIRTEXE | xcv200efg456 | 237KB | 7MB | 0.117s |
| VIRTEXE | xcv200epq240 | 237KB | 7MB | 0.117s |
| VIRTEXE | xcv2600efg1156 | 1073KB | 64MB | 0.774s |
| VIRTEXE | xcv300ebg352 | 281KB | 9MB | 0.137s |
| VIRTEXE | xcv300ebg432 | 281KB | 9MB | 0.141s |
| VIRTEXE | xcv300efg256 | 281KB | 9MB | 0.138s |
| VIRTEXE | xcv300efg456 | 281KB | 9MB | 0.138s |
| VIRTEXE | xcv300epq240 | 281KB | 9MB | 0.140s |
| VIRTEXE | xcv3200efg1156 | 1304KB | 81MB | 0.949s |
| VIRTEXE | xcv400ebg432 | 335KB | 13MB | 0.192s |
| VIRTEXE | xcv400ebg560 | 335KB | 13MB | 0.202s |
| VIRTEXE | xcv400efg676 | 335KB | 13MB | 0.189s |
| VIRTEXE | xcv400epq240 | 335KB | 13MB | 0.200s |
| VIRTEXE | xcv405ebg560 | 388KB | 14MB | 0.202s |
| VIRTEXE | xcv405efg676 | 388KB | 14MB | 0.203s |
| VIRTEXE | xcv50ecs144 | 152KB | 2MB | 0.079s |
| VIRTEXE | xcv50efg256 | 152KB | 2MB | 0.078s |
| VIRTEXE | xcv50epq240 | 152KB | 2MB | 0.079s |
| VIRTEXE | xcv600ebg432 | 413KB | 19MB | 0.258s |
| VIRTEXE | xcv600ebg560 | 413KB | 19MB | 0.245s |
| VIRTEXE | xcv600efg676 | 413KB | 19MB | 0.258s |
| VIRTEXE | xcv600efg680 | 413KB | 19MB | 0.258s |
| VIRTEXE | xcv600efg900 | 413KB | 19MB | 0.259s |
| VIRTEXE | xcv600ehq240 | 413KB | 19MB | 0.246s |
| VIRTEXE | xcv812ebg560 | 634KB | 26MB | 0.349s |
| VIRTEXE | xcv812efg900 | 634KB | 26MB | 0.344s |

An XDL-based import/export capability has also been created and used with Virtex 6 devices as a part of Travis Haroldsen’s PhD work but that path is not being released, documented, or supported.

There is actually a third category – code that functions but which has not been documented yet. For example, the first BYU version of RS2 code had the subsite package but was designed to work with XDL files. Thus, it contained code to pack and unpack XDL files into the RapidSmith Cell/BEL data structure.